Hardware Design Specification

I²C Master Controller — Synthesizable Verilog RTL

| Document Number | HDS-I2C-001 |
| --- | --- |
| Revision | A.1 |
| Status | Released for Review |
| Date | March 2026 |
| Author | RTL Design Team |
| Reviewer | Architecture Review Board |
| Compliance | NXP UM10204 Rev. 7 |

Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | Jan 2026 | RTL Team | Initial draft — architecture definition |
| 0.5 | Feb 2026 | RTL Team | Added FSM states, timing tables, register map |
| A.0 | Feb 2026 | RTL Team | First complete draft submitted for review |
| A.1 | Mar 2026 | RTL Team | Incorporated ARB review comments; FM+ added |

Table of Contents

1  Overview  ........  4

2  Functional Requirements  ........  5

3  Interface Definition  ........  7

4  Internal Architecture  ........  10

5  Timing Specification  ........  13

6  Error Handling  ........  15

7  Register Map  ........  16

8  Constraints and Assumptions  ........  19

9  Verification Plan  ........  20

10  Compliance Reference  ........  22

1  Overview

1.1  Purpose

This document defines the hardware design specification (HDS) for a synthesizable Verilog RTL implementation of an I²C Master Controller. The controller provides a standards-compliant two-wire serial interface suitable for integration into FPGA-based system-on-chip (SoC) designs and full-custom ASIC tape-outs.

The I²C (Inter-Integrated Circuit) protocol, developed by NXP Semiconductors, enables multi-device communication over a shared two-wire bus using only a serial data line (SDA) and a serial clock line (SCL). This controller targets low-to-medium bandwidth peripheral communication, including sensors, EEPROMs, power management ICs, display controllers, and real-time clocks.

1.2  Target Use Cases

FPGA prototyping and production (Xilinx UltraScale+, Intel Agilex, Lattice ECP5)

ASIC implementation at 28 nm, 22 nm, or 16 nm process nodes

Embedded SoC subsystems requiring I²C peripheral connectivity

Data-acquisition systems with I²C sensors and ADCs

Low-power IoT edge nodes requiring configurable bus speed

1.3  Specification Compliance

The controller complies with the NXP I²C-bus specification and user manual UM10204 Revision 7 (October 2021). All timing parameters, electrical characteristics, and protocol behaviors referenced in this document are derived from that specification.

| Mode | Bit Rate | Compliance Level |
| --- | --- | --- |
| Standard-mode (Sm) | ≤ 100 kbit/s | Required |
| Fast-mode (Fm) | ≤ 400 kbit/s | Required |
| Fast-mode Plus (Fm+) | ≤ 1 Mbit/s | Optional (register-selectable) |
| High-speed mode (Hs) | ≤ 3.4 Mbit/s | Out of scope |
| Ultra-fast mode (UFm) | ≤ 5 Mbit/s | Out of scope |

NOTE: Fm+ operation requires open-drain I/O cells capable of sinking up to 20 mA. Verify I/O library support before enabling this mode in ASIC context.

1.4  Document Scope

This specification covers the RTL micro-architecture, port definitions, internal FSM, register map, timing requirements, error handling, and verification strategy. Physical implementation (floorplanning, place-and-route, timing closure) is outside the scope of this document.

2  Functional Requirements

2.1  Operating Mode

The controller operates exclusively as an I²C Master. Slave mode is not implemented in this revision. A single instantiation of this block drives the bus and initiates all transactions. Multi-master arbitration (Section 2.7) is supported to allow co-existence with other masters on the same bus segment.

2.2  Addressing

7-bit addressing: Default operational mode. Address occupies bits [7:1] of the first byte transmitted after START; bit [0] is the R/W direction flag.

10-bit addressing: Enabled via the ADDR10 control register bit. First byte is the reserved pattern 11110XX where XX are the two MSBs of the 10-bit address; second byte carries the remaining 8 address bits.

General Call address (0x00): Detected and passed to the host; the controller does not respond autonomously.

NOTE: The addressing mode (7-bit vs 10-bit) is selected per-transaction via the CTRL register ADDR10 bit.

2.3  Bus Condition Generation

The controller hardware generates and detects the following I²C bus conditions:

| Condition | Description | Generation Method |
| --- | --- | --- |
| START (S) | SDA falls while SCL is HIGH | Assert SDA low, then assert SCL low |
| REPEATED START (Sr) | START without preceding STOP | Release SCL/SDA high, re-assert START sequence |
| STOP (P) | SDA rises while SCL is HIGH | SCL high, then release SDA high |
| ACK | Receiver pulls SDA low during 9th clock | Drive SDA=0 during ACK phase (when master reads) |
| NACK | Receiver releases SDA during 9th clock | Release SDA=1 during ACK phase |

2.4  ACK / NACK Handling

Write Transactions: Controller monitors SDA during the 9th clock pulse to detect slave ACK or NACK. A NACK sets the NACK_DET status flag and optionally generates an interrupt.

Read Transactions: Controller drives ACK after each received byte (except the final byte, where it drives NACK to signal end-of-transfer to the slave).

The number of bytes to read before issuing a NACK is specified in the BYTE_COUNT register field.

2.5  Clock Stretching

The controller supports clock stretching as both a generator and a responder. A slave device may hold SCL low to pause the transfer. The controller detects a stretched SCL by sampling the SCL input pin after asserting SCL high; if SCL remains low, the controller enters a WAIT state and polls until SCL is released. A programmable stretch timeout (STRETCH_TO field) prevents indefinite stalls.

2.6  Read and Write Transactions

The controller supports the following transaction types:

Master Write: START → Address+W → ACK → Data[0..N-1] → ACK per byte → STOP

Master Read:  START → Address+R → ACK → Data[0..N-1] (master ACKs all but last) → NACK → STOP

Combined (Write-then-Read): START → Address+W → Data → Sr → Address+R → Data → STOP

Burst Write: N data bytes transferred after a single address phase, limited by BYTE_COUNT[7:0] (max 255 bytes per transaction without software re-trigger).

Burst Read:  N consecutive bytes received; master ACKs bytes 0..N-2, NACKs byte N-1.

2.7  Multi-Master Arbitration

When multiple masters attempt simultaneous bus access, the controller implements bitwise arbitration per the I²C specification:

After driving SDA low, the controller samples the SDA input at the rising edge of SCL.

If the sampled SDA is high (another master drove it high), arbitration is lost.

On arbitration loss, the controller immediately releases SDA and SCL, sets the ARB_LOST flag, asserts the interrupt line, and transitions to IDLE.

No automatic retry is performed; software must re-initiate the transaction.

NOTE: Arbitration is only meaningful in multi-master environments. Single-master systems may leave this detection active with no functional impact.

2.8  Bus Busy Detection

The BUSY flag is set when the controller detects a START condition on the bus (SDA falling while SCL is high) and is cleared when a STOP condition is detected. Software must not initiate a new transaction while BUSY is asserted.

2.9  Interrupt Support

The controller provides a single maskable interrupt output. Interrupt sources include:

| Interrupt Source | Flag Bit | Description |
| --- | --- | --- |
| Transfer Complete | INT_TC | All bytes transferred successfully, STOP issued |
| NACK Received | INT_NACK | Slave responded with NACK |
| Arbitration Lost | INT_ARB | Arbitration lost; bus released |
| Bus Error | INT_BERR | Unexpected START/STOP detected mid-transfer |
| Stretch Timeout | INT_STTO | Clock stretch exceeded STRETCH_TO limit |
| TX FIFO Empty | INT_TXFE | Transmit FIFO empty during burst write |
| RX FIFO Full | INT_RXFF | Receive FIFO full; data may be lost |

3  Interface Definition

3.1  Top-Level Module Declaration

module i2c_master #(

parameter SYS_CLK_HZ    = 50_000_000,  // System clock frequency (Hz)

parameter SCL_HZ        = 100_000,      // Default SCL frequency (Hz)

parameter FIFO_DEPTH    = 16,           // TX/RX FIFO depth (power of 2)

parameter ADDR_WIDTH    = 8,            // APB address bus width

parameter DATA_WIDTH    = 32            // APB data bus width

) (

// ─── Clocks and Resets ─────────────────────────────────────────

input  wire        pclk,                // System / APB clock

input  wire        presetn,             // Active-low synchronous reset

// ─── APB Register Interface ────────────────────────────────────

input  wire        psel,

input  wire        penable,

input  wire        pwrite,

input  wire [ADDR_WIDTH-1:0]  paddr,

input  wire [DATA_WIDTH-1:0]  pwdata,

output wire [DATA_WIDTH-1:0]  prdata,

output wire        pready,

output wire        pslverr,

// ─── I²C Bus (open-drain) ──────────────────────────────────────

input  wire        scl_i,               // SCL input (from pad)

output wire        scl_o,               // SCL drive enable (active-low OD)

output wire        scl_oen,             // SCL output enable (0 = drive low)

input  wire        sda_i,               // SDA input (from pad)

output wire        sda_o,               // SDA drive (always 0; OD pull-down)

output wire        sda_oen,             // SDA output enable (0 = drive low)

// ─── Interrupt ─────────────────────────────────────────────────

output wire        irq                  // Active-high interrupt output

);

NOTE: Open-drain I/O is modelled using separate _o (always 0) and _oen (output-enable) signals. The I/O cell or pad wrapper must implement: pad = oen ? 1'bz : 0.

3.2  Port Descriptions

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| pclk | In | 1 | Primary clock. All flops are synchronous to this clock. Supports up to 200 MHz. |
| presetn | In | 1 | Active-low synchronous reset. All state machines and registers are reset on assertion. |
| psel | In | 1 | APB peripheral select. Qualifies all APB transactions. |
| penable | In | 1 | APB enable. Asserted on the second (and subsequent) cycles of an APB transfer. |
| pwrite | In | 1 | APB transfer direction: 1 = Write, 0 = Read. |
| paddr | In | 8 | APB address bus selecting the target register (word-aligned). |
| pwdata | In | 32 | APB write data bus. |
| prdata | Out | 32 | APB read data bus. Valid when psel=1, pwrite=0. |
| pready | Out | 1 | APB ready signal. Tie high (combinatorially) for zero-wait-state accesses. |
| pslverr | Out | 1 | APB slave error. Asserted when an undefined register is accessed. |
| scl_i | In | 1 | SCL line sampled from I/O pad. Used for clock-stretch detection and arbitration. |
| scl_o | Out | 1 | SCL drive value. Tied to logic 0; drive is gated by scl_oen. |
| scl_oen | Out | 1 | SCL output enable (0 = drive low, 1 = tri-state/release to pull-up). |
| sda_i | In | 1 | SDA line sampled from I/O pad. Used for arbitration and data reception. |
| sda_o | Out | 1 | SDA drive value. Tied to logic 0. |
| sda_oen | Out | 1 | SDA output enable (0 = drive low, 1 = tri-state/release to pull-up). |
| irq | Out | 1 | Interrupt request. Active-high, level-sensitive. Cleared by writing 1 to the active INT_STATUS bit. |

3.3  Clock and Reset Requirements

The controller uses a single clock domain (pclk). The system clock frequency must be at least 10× the desired SCL frequency to achieve adequate timing resolution. Recommended minimum clock frequencies are listed below.

| SCL Mode | SCL Freq. | Min pclk | Recommended pclk |
| --- | --- | --- | --- |
| Standard-mode | 100 kHz | 1 MHz | ≥ 10 MHz |
| Fast-mode | 400 kHz | 4 MHz | ≥ 20 MHz |
| Fast-mode Plus | 1 MHz | 10 MHz | ≥ 50 MHz |

Reset (presetn) is synchronous and active-low. The reset must be asserted for a minimum of two pclk rising edges. All output enables default to 1 (tri-state) after reset, ensuring no spurious bus activity.

3.4  APB Register Interface

The controller uses an AMBA 3 APB (Advanced Peripheral Bus) interface for register access. The interface is zero-wait-state for all register reads and writes (pready is always driven high). The complete register map is described in Section 7.

4  Internal Architecture

4.1  Block Diagram (Textual)

┌─────────────────────────────────────────────┐

APB Bus ───────►  APB Slave Interface & Register Block        │

│  (CTRL, STATUS, ADDR, DATA, CLKDIV, INT)     │

└──────────────┬──────────────────────────────┘

│ Control/Status signals

┌──────────────▼──────────────────────────────┐

TX FIFO  ──────►                                             ◄── sda_i / scl_i

RX FIFO  ◄──────  Master FSM (i2c_master_fsm)               │

│  • State: IDLE, START, ADDR, DATA, ACK ...  │

└──────────────┬──────────────────────────────┘

│

┌───────────────────┼───────────────────────┐

│                   │                       │

┌─────────▼──────┐  ┌─────────▼──────┐  ┌────────────▼──────┐

│  Clock Divider │  │  Shift Register│  │  SDA/SCL OD Ctrl  │

│  (SCL gen)     │  │  (8-bit TX/RX) │  │  (drive logic)    │

└─────────┬──────┘  └────────────────┘  └─────┬──────┬──────┘

│                                    │      │

scl_oen                             sda_oen  scl_oen

└────────────────────────────────────┘      │

I²C Bus                     │

4.2  Sub-Block Descriptions

4.2.1  APB Slave Interface

Decodes APB transactions and provides read/write access to the internal register file. Implements pslverr for undefined address ranges. All register writes are synchronous; reads are combinatorial.

4.2.2  TX / RX FIFOs

Synchronous FIFOs of parameterizable depth (default 16 entries × 8 bits). TX FIFO feeds the shift register during write and combined transactions. RX FIFO captures received bytes. Overflow/underflow conditions are flagged in the STATUS register and can trigger interrupts.

4.2.3  Clock Divider (SCL Generator)

A 16-bit down-counter derives SCL timing from pclk. The divisor is loaded from the CLKDIV register (default configured at synthesis time from SYS_CLK_HZ and SCL_HZ parameters). Two phases are maintained:

SCL_HIGH_CNT = (pclk / SCL_HZ) × duty_high — configurable via CLKDIV_H field

SCL_LOW_CNT  = (pclk / SCL_HZ) × duty_low  — configurable via CLKDIV_L field

The FSM advances state only on the SCL phase tick (clk_tick_h or clk_tick_l). The clock divider is halted when the FSM is in IDLE or WAIT states.

4.2.4  Shift Register

An 8-bit parallel-load / serial-shift register transmits and receives data. For writes, the register is loaded from the TX FIFO on entry to the DATA state and shifted MSB-first on each SCL rising edge. For reads, bits are captured on each SCL rising edge; the complete byte is written to the RX FIFO on exit from the DATA state.

4.2.5  Bit Counter

A 4-bit counter (bit_cnt[3:0]) tracks the current bit position within a byte (counts 7 down to 0) plus the ACK bit (position 8). The counter is reloaded to 7 on each entry to DATA state. Reaching 0 triggers transition to ACK state. A separate byte counter (byte_cnt[7:0]) tracks remaining bytes in a burst.

4.2.6  SDA / SCL Open-Drain Control

Both bus lines are modelled as open-drain. The controller never drives a line high; it either asserts the output-enable low (drives line to 0) or deasserts it (releases the line to the pull-up resistor). The control signals are:

assign scl_o   = 1'b0;             // Always 0; drive gated by scl_oen

assign scl_oen = ~scl_drive;        // scl_drive=1 → pull SCL low

assign sda_o   = 1'b0;             // Always 0; drive gated by sda_oen

assign sda_oen = ~sda_drive;        // sda_drive=1 → pull SDA low

4.3  Master FSM

The controller FSM is a Moore machine with synchronous state transitions. All outputs are registered to prevent glitches on the bus. The FSM operates on ticks from the clock divider, not on raw pclk edges.

| State | Encoding | Description |
| --- | --- | --- |
| IDLE | 4'h0 | Bus idle; awaiting software START command (GO bit set) or detecting bus busy. |
| BUS_BUSY | 4'h1 | Bus busy detected; waiting for STOP condition before attempting access. |
| GEN_START | 4'h2 | Generating START: drives SDA low while SCL is high. Advances to GEN_ADDR on tick. |
| GEN_RSTART | 4'h3 | Generating REPEATED START: releases SDA/SCL, then re-asserts START sequence. |
| GEN_ADDR | 4'h4 | Shifts 7-bit (or 10-bit first byte) address + R/W bit onto SDA MSB-first. |
| GEN_ADDR2 | 4'h5 | Shifts 10-bit address second byte (skipped in 7-bit mode). |
| CHECK_ACK | 4'h6 | Samples SDA during 9th SCL pulse; branches to DATA_TX/RX on ACK, or ERROR on NACK. |
| DATA_TX | 4'h7 | Shifts TX FIFO byte onto SDA MSB-first; transitions to ACK_WAIT after 8 bits. |
| ACK_WAIT | 4'h8 | Releases SDA; samples SDA at SCL high for slave ACK. Loads next byte or proceeds to STOP. |
| DATA_RX | 4'h9 | Samples SDA at SCL rising edge; shifts into RX shift register; 8 bits → RX FIFO. |
| GEN_ACK | 4'hA | Drives SDA low (ACK) or releases (NACK) during 9th SCL pulse after DATA_RX. |
| GEN_STOP | 4'hB | Generates STOP: SCL high, then SDA rises. Returns to IDLE. |
| WAIT_STRETCH | 4'hC | Monitors SCL_I; remains here while slave stretches clock. Timeout counter active. |
| ARB_LOST | 4'hD | Arbitration lost; releases bus, sets ARB_LOST flag, returns to IDLE after one cycle. |
| ERROR | 4'hE | Bus or protocol error; asserts interrupt, drives STOP, returns to IDLE. |

4.4  Key FSM Transitions

IDLE         → GEN_START     : CTRL.GO=1 && !STATUS.BUSY

IDLE         → BUS_BUSY      : START detected on bus (sda_i=0, scl_i=1)

BUS_BUSY     → IDLE          : STOP detected on bus (sda_i=1, scl_i=1)

GEN_START    → GEN_ADDR      : START timing complete

GEN_ADDR     → CHECK_ACK     : All address bits shifted (bit_cnt==0)

CHECK_ACK    → DATA_TX       : sda_i==0 (ACK) && CTRL.RW==WRITE

CHECK_ACK    → DATA_RX       : sda_i==0 (ACK) && CTRL.RW==READ

CHECK_ACK    → ERROR         : sda_i==1 (NACK)

DATA_TX      → ACK_WAIT      : bit_cnt==0

ACK_WAIT     → DATA_TX       : sda_i==0 && byte_cnt>0

ACK_WAIT     → GEN_RSTART    : sda_i==0 && byte_cnt==0 && CTRL.RSTART

ACK_WAIT     → GEN_STOP      : sda_i==0 && byte_cnt==0 && !CTRL.RSTART

ACK_WAIT     → ERROR         : sda_i==1 (unexpected NACK mid-burst)

DATA_RX      → GEN_ACK       : bit_cnt==0

GEN_ACK      → DATA_RX       : byte_cnt>0 (master sends ACK)

GEN_ACK      → GEN_STOP      : byte_cnt==0 (master sends NACK)

GEN_STOP     → IDLE          : STOP timing complete

WAIT_STRETCH → <prev_state>  : scl_i==1 (slave released SCL)

WAIT_STRETCH → ERROR         : stretch_timeout_cnt==0

Any State    → ARB_LOST      : sda_i==1 while driving sda_oen=0

5  Timing Specification

5.1  I²C Timing Parameters

The following table lists the AC timing parameters as defined in NXP UM10204 Table 10. All values are minimum requirements; the controller implementation must meet or exceed these.

| Symbol | Parameter | Sm Min (µs) | Fm Min (µs) | Fm+ Min (µs) | Unit |
| --- | --- | --- | --- | --- | --- |
| f_SCL | SCL clock frequency | 0 | 0 | 0 | kHz max: 100/400/1000 |
| t_HD;STA | Hold time START condition | 4.0 | 0.6 | 0.26 | µs |
| t_LOW | SCL low period | 4.7 | 1.3 | 0.5 | µs |
| t_HIGH | SCL high period | 4.0 | 0.6 | 0.26 | µs |
| t_SU;STA | Setup time repeated START | 4.7 | 0.6 | 0.26 | µs |
| t_HD;DAT | Data hold time | 0 | 0 | 0 | µs (min 0 ns) |
| t_SU;DAT | Data setup time | 250 | 100 | 50 | ns |
| t_r | SDA/SCL rise time | 1000 | 300 | 120 | ns (max) |
| t_f | SDA/SCL fall time | 300 | 300 | 120 | ns (max) |
| t_SU;STO | Setup time STOP condition | 4.0 | 0.6 | 0.26 | µs |
| t_BUF | Bus free time between STOP/START | 4.7 | 1.3 | 0.5 | µs |
| t_SP | Pulse width of spikes suppressed | — | 50 | 50 | ns (max) |

NOTE: Rise/fall times (t_r, t_f) depend on external pull-up resistors and bus capacitance. RTL implementation assumes ideal transitions; timing closure constraints must account for pad characteristics.

5.2  SCL Clock Generation Timing

The SCL clock divider generates asymmetric duty cycles to satisfy both t_LOW and t_HIGH simultaneously. With a 50 MHz system clock:

| Mode | SCL_HZ | pclk cycles/period | HIGH cycles (min) | LOW cycles (min) |
| --- | --- | --- | --- | --- |
| Standard-mode | 100 kHz | 500 | 200 (4.0 µs @ 50 MHz) | 235 (4.7 µs @ 50 MHz) |
| Fast-mode | 400 kHz | 125 | 30 (0.6 µs @ 50 MHz) | 65 (1.3 µs @ 50 MHz) |
| Fm+ | 1 MHz | 50 | 13 (0.26 µs @ 50 MHz) | 25 (0.5 µs @ 50 MHz) |

5.3  Timing Diagrams (Described)

5.3.1  START Condition

SCL: ─────────┐         ┌────────────────

└─────────┘

HIGH ──────┐

SDA:            ↓   └────────────────────

t_HD;STA

SDA falls while SCL is HIGH → START detected

Controller must hold SDA low for t_HD;STA before pulling SCL low

5.3.2  Data Transfer Bit

t_SU;DAT   t_HD;DAT

◄──────►   ◄──────►

SCL:  ────────────┐          ┌─────────────

t_LOW       └──────────┘

↑ sample point

SDA:  ──XXXXXX────────────────────────XXXXX

data stable during SCL HIGH

5.3.3  STOP Condition

SCL:  ─────────────────────────────────────

HIGH (must remain high)

t_SU;STO

◄──────►

SDA:  ──────────┐                 ┌────────

└─────────────────┘

SDA rises while SCL HIGH → STOP

5.4  Glitch Filter

The controller implements a 2-stage synchronizer followed by a 3-clock-cycle majority filter on both SDA_I and SCL_I to suppress glitches and metastability. This filter introduces a 5-cycle latency on input transitions, which is accounted for in the FSM hold-time calculations.

6  Error Handling

6.1  Error Conditions

| Error Condition | Detection Method | Recovery Action | Status Flag |
| --- | --- | --- | --- |
| Bus Busy | START detected (SDA_I low, SCL_I high) before controller initiated transfer | Wait for STOP; set BUSY flag; do not initiate | STATUS.BUSY |
| Arbitration Loss | sda_i=1 sampled when controller is driving sda_oen=0 | Release bus immediately; assert ARB_LOST; IRQ; return to IDLE | STATUS.ARB_LOST |
| NACK on Address | sda_i=1 during ACK phase after address transmission | Generate STOP; set NACK_DET; IRQ | STATUS.NACK_DET |
| NACK on Data | sda_i=1 during ACK phase after data byte transmission | Generate STOP; set NACK_DET; IRQ | STATUS.NACK_DET |
| Clock Stretch Timeout | SCL_I remains low after controller releases SCL beyond STRETCH_TO cycles | Generate STOP; set STTO; IRQ | STATUS.STTO |
| Bus Error (unexpected START/STOP) | START or STOP detected during data transfer phase | Reset shift register; set BERR; IRQ; attempt STOP or idle | STATUS.BERR |
| TX FIFO Underrun | Shift register requests next byte but FIFO is empty during burst | Insert 0xFF; set TXUR; IRQ; complete byte; generate STOP | STATUS.TXUR |
| RX FIFO Overrun | RX FIFO full when shift register completes byte | Discard received byte; set RXOR; IRQ; continue (data integrity lost) | STATUS.RXOR |

6.2  Error Priority

When multiple error conditions occur simultaneously, the following priority order applies (highest to lowest): Bus Error > Arbitration Loss > Clock Stretch Timeout > NACK > FIFO Underrun/Overrun. The highest priority flag is set; lower priority flags may also be set but do not change the recovery sequence.

6.3  Software Recovery Procedure

Read STATUS register to identify error condition.

Write 1 to the relevant INT_STATUS bit to clear the interrupt.

Wait until STATUS.BUSY is deasserted (bus free).

If STATUS.BERR is set, write CTRL.SWRST=1 to perform a soft reset of the FSM and FIFOs.

Flush TX/RX FIFOs by writing FIFO_CTRL.TXFLUSH=1 and FIFO_CTRL.RXFLUSH=1.

Re-configure address, data, and byte count registers.

Write CTRL.GO=1 to re-initiate the transaction.

7  Register Map

7.1  Register Summary

| Offset | Register | Access | Reset Value | Description |
| --- | --- | --- | --- | --- |
| 0x00 | CTRL | R/W | 0x0000_0000 | Control register |
| 0x04 | STATUS | R/C | 0x0000_0000 | Status register (write-1-to-clear flags) |
| 0x08 | ADDR | R/W | 0x0000_0000 | Target slave address |
| 0x0C | DATA_TX | W | 0x0000_00FF | TX FIFO write port |
| 0x10 | DATA_RX | R | 0x0000_0000 | RX FIFO read port |
| 0x14 | BYTE_COUNT | R/W | 0x0000_0001 | Number of bytes to transfer |
| 0x18 | CLKDIV | R/W | param-based | SCL clock divider (high/low counts) |
| 0x1C | INT_MASK | R/W | 0x0000_0000 | Interrupt enable mask |
| 0x20 | INT_STATUS | R/C | 0x0000_0000 | Interrupt status (write-1-to-clear) |
| 0x24 | FIFO_CTRL | R/W | 0x0000_0000 | FIFO control (flush, level) |
| 0x28 | FIFO_STAT | R | 0x0000_0000 | FIFO fill levels |
| 0x2C | TIMEOUT | R/W | 0x0000_FFFF | Clock stretch timeout count |
| 0x30 | VERSION | R | 0x0001_0100 | IP version (major.minor.patch) |

7.2  Register Bit Definitions

7.2.1  CTRL — Control Register (0x00)

| Bits | Field | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [0] | GO | W | 0 | Write 1 to start a transfer. Self-clears when transfer begins. Read returns 0. |
| [1] | STOP | W | 0 | Write 1 to force STOP condition. Aborts current transfer. |
| [2] | RW | R/W | 0 | 0 = Write transaction; 1 = Read transaction. |
| [3] | RSTART | R/W | 0 | 1 = Issue REPEATED START after write phase (combined transfer). |
| [4] | ADDR10 | R/W | 0 | 1 = Use 10-bit addressing mode. |
| [5] | FM_PLUS | R/W | 0 | 1 = Enable Fast-mode Plus (1 MHz) timing. |
| [6] | SWRST | W | 0 | Software reset: clears FSM and FIFOs. Self-clears after one cycle. |
| [7] | IE | R/W | 0 | Global interrupt enable. |
| [31:8] | RSVD | R | 0 | Reserved. Write 0; reads return 0. |

7.2.2  STATUS — Status Register (0x04)

| Bits | Field | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [0] | BUSY | R | 0 | 1 = Bus is busy (START detected, no STOP seen). |
| [1] | ACTIVE | R | 0 | 1 = Transfer in progress. |
| [2] | ARB_LOST | R/C | 0 | 1 = Arbitration lost. Write 1 to clear. |
| [3] | NACK_DET | R/C | 0 | 1 = NACK received from slave. |
| [4] | BERR | R/C | 0 | 1 = Bus error (unexpected START/STOP). |
| [5] | STTO | R/C | 0 | 1 = Clock stretch timeout expired. |
| [6] | TXUR | R/C | 0 | 1 = TX FIFO underrun during burst. |
| [7] | RXOR | R/C | 0 | 1 = RX FIFO overrun; data discarded. |
| [8] | TC | R/C | 0 | 1 = Transfer complete (STOP issued). |
| [31:9] | RSVD | R | 0 | Reserved. |

7.2.3  ADDR — Slave Address Register (0x08)

| Bits | Field | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [6:0] | ADDR7 | R/W | 0x00 | 7-bit slave address (used when CTRL.ADDR10=0). |
| [9:0] | ADDR10 | R/W | 0x000 | 10-bit slave address (used when CTRL.ADDR10=1). Bits [6:0] map to [6:0]; bits [9:7] map to [9:7]. |
| [31:10] | RSVD | R | 0 | Reserved. |

7.2.4  CLKDIV — Clock Divider Register (0x18)

| Bits | Field | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [15:0] | CLKDIV_L | R/W | param | SCL LOW duration in pclk cycles minus 1. Must satisfy t_LOW. |
| [31:16] | CLKDIV_H | R/W | param | SCL HIGH duration in pclk cycles minus 1. Must satisfy t_HIGH. |

NOTE: At synthesis, CLKDIV reset values are computed as: CLKDIV_L = (SYS_CLK_HZ / SCL_HZ) × 0.53 - 1 (53% low duty cycle provides margin for both Sm and Fm modes).

7.2.5  FIFO_CTRL — FIFO Control Register (0x24)

| Bits | Field | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [0] | TXFLUSH | W | 0 | Write 1 to flush TX FIFO. Self-clears. |
| [1] | RXFLUSH | W | 0 | Write 1 to flush RX FIFO. Self-clears. |
| [7:4] | TX_AFULL_THR | R/W | 0xC | TX almost-full threshold (entries). Generates INT_TXFE when below. |
| [11:8] | RX_AFULL_THR | R/W | 0xC | RX almost-full threshold. Generates INT_RXFF when at or above. |
| [31:12] | RSVD | R | 0 | Reserved. |

8  Constraints and Assumptions

8.1  I/O Cell Requirements

The I²C specification mandates open-drain (open-collector) bus topology. The RTL model does not include I/O cell primitives; the integrating SoC or FPGA top-level must instantiate appropriate I/O buffers.

FPGA: Use IOBUF or equivalent primitive configured in open-drain mode. For Xilinx, set DRIVE=4 and SLEW=SLOW; instantiate PULLUP on SCL/SDA.

ASIC: Use an open-drain I/O cell with independent input and output-enable ports, matching the scl_o/scl_oen interface convention.

The integrator must ensure the I/O cell input path (scl_i, sda_i) is not inverted relative to the bus signal.

8.2  Pull-up Resistors

External pull-up resistors are required on SCL and SDA. Recommended values:

| Mode | Max Bus Capacitance | Min R_pull (kΩ) | Max R_pull (kΩ) |
| --- | --- | --- | --- |
| Standard-mode | 400 pF | 1.0 | 10.0 |
| Fast-mode | 400 pF | 0.43 | 3.0 |
| Fast-mode Plus | 550 pF | 0.1 | 1.0 |

NOTE: For Fm+, active pull-up current sources may be required. Consult NXP UM10204 Section 7.1 for electrical specifications.

8.3  Clocking Assumptions

A single synchronous clock domain is assumed. No CDC (clock domain crossing) logic is included.

The system clock must be glitch-free and have a clean PLL-sourced waveform.

The clock must be running and stable before presetn is deasserted.

APB pclk and the I²C controller clock are the same clock in this design.

8.4  Bus Capacitance

RTL simulation assumes ideal (zero-rise-time) I/O. Post-layout timing must verify that SCL/SDA edge rates meet the t_r / t_f requirements for the target bus capacitance. Designs with heavily loaded buses (>100 pF) should add the capacitance to the STA constraint file.

8.5  Synthesis Constraints

Set a false path from presetn to all timing paths (synchronous reset, no recovery/removal check needed on the reset pin itself).

Set output delay constraints on scl_oen and sda_oen relative to pclk, accounting for I/O propagation and t_SU;DAT.

Set input delay constraints on scl_i and sda_i relative to pclk, accounting for I/O propagation.

The irq output should have a max output delay of one pclk cycle.

9  Verification Plan

9.1  Verification Strategy

Verification is conducted using a UVM-based testbench targeting 100% functional coverage and > 95% code coverage (line, branch, toggle). Directed tests cover specification requirements; randomised constrained tests exercise corner cases and protocol interactions. Formal verification is used to prove FSM deadlock-freedom and reset correctness.

9.2  Directed Test Plan

| Test ID | Test Name | Description | Pass Criteria |
| --- | --- | --- | --- |
| DT-01 | Single-byte Write | 7-bit address + 1 data byte + STOP | Correct SDA sequence; ACK sampled; TC interrupt |
| DT-02 | Single-byte Read | 7-bit address+R + 1 byte RX + NACK + STOP | Byte in RX FIFO; TC interrupt |
| DT-03 | Burst Write 16 bytes | 7-bit addr + 16 data bytes + STOP | All bytes sent in order; no TXUR |
| DT-04 | Burst Read 16 bytes | 7-bit addr+R + 16 bytes received | All bytes in RX FIFO in order |
| DT-05 | Combined Wr-then-Rd | Write 2 bytes, Sr, Read 3 bytes, STOP | Correct RSTART timing; both phases complete |
| DT-06 | 10-bit Address Write | 10-bit addr + data + STOP | Two-byte address sequence on bus |
| DT-07 | NACK on Address | Slave model returns NACK after address phase | NACK_DET set; STOP generated; IRQ asserted |
| DT-08 | NACK Mid-burst | Slave NACKs on 4th byte of 8-byte write | NACK_DET set; STOP after 4th byte |
| DT-09 | Clock Stretch | Slave stretches SCL for 50 µs | Controller stalls; resumes after stretch; TC flag |
| DT-10 | Stretch Timeout | Slave holds SCL low beyond TIMEOUT value | STTO set; STOP generated; IRQ |
| DT-11 | Bus Busy Detection | Another master initiates START before our controller | BUSY set; our transfer deferred |
| DT-12 | Arbitration Loss | Two masters simultaneously transmit different data | Losing master sets ARB_LOST; releases bus |
| DT-13 | Standard-mode 100 kHz | Full write transaction at 100 kHz SCL | All timing parameters within UM10204 Table 10 |
| DT-14 | Fast-mode 400 kHz | Full write transaction at 400 kHz SCL | All Fm timing parameters met |
| DT-15 | Fm+ 1 MHz | Full write transaction at 1 MHz SCL | All Fm+ timing parameters met |
| DT-16 | Soft Reset During Xfer | Assert CTRL.SWRST mid-transfer | Bus released; FIFOs cleared; IDLE state reached |
| DT-17 | TX FIFO Underrun | BYTE_COUNT > TX FIFO entries at transfer start | TXUR set; STOP generated; IRQ |
| DT-18 | RX FIFO Overrun | RX FIFO not drained; 17 bytes received into depth-16 FIFO | RXOR set; last byte discarded; IRQ |
| DT-19 | Repeated START sequence | Combined transfer verified on bus analyser model | Sr timing: t_SU;STA met; no STOP between phases |
| DT-20 | General Call (0x00) | Master sends General Call address | Address 0x00 transmitted; ACK from slave model |

9.3  Corner Cases

Zero-length transfer (BYTE_COUNT = 0): Controller should generate START + Address + STOP with no data phase.

Back-to-back transactions (GO asserted immediately after TC): Verify t_BUF bus-free time is respected before new START.

Address 0x7F (broadcast): Verify address bits shifted correctly without aliasing to 10-bit first-byte pattern.

Maximum burst (BYTE_COUNT = 255): Verify byte_cnt wraps correctly and TC fires after 255th ACK.

Reset during arbitration-loss handling: Verify no spurious drive on SDA/SCL after reset.

CLKDIV = 0: Verify design does not hang; synthesizer should assert an assertion/error or clamp to minimum safe value.

Simultaneous STTO and BERR: Verify priority logic sets correct flag and IRQ fires once.

9.4  Bus Contention Scenarios

Two-master arbitration: Master A sends 0xA5, Master B sends 0x5A to the same address. Master B should lose on first bit mismatch (MSB=0 vs 1).

Slave holds SCL low immediately after START: Verify WAIT_STRETCH entry within one SCL period.

Glitch on SDA during address phase: Verify glitch filter prevents spurious bit flip; spike < t_SP must be suppressed.

Power-on with bus held low: Verify controller enters BUS_BUSY and waits for STOP before proceeding.

9.5  Functional Coverage Points

| Coverage Group | Coverpoint / Cross |
| --- | --- |
| cg_mode | All SCL modes (Sm, Fm, Fm+) hit at least once per regression |
| cg_addr | 7-bit and 10-bit addressing both exercised; address values 0x00, 0x7F, random |
| cg_rw | Read-only, Write-only, and Combined transactions |
| cg_burst | Burst lengths: 1, 2, 8, 16, 64, 255 |
| cg_ack | ACK and NACK on every possible byte position within a burst |
| cg_errors | All error flags (ARB_LOST, NACK_DET, BERR, STTO, TXUR, RXOR) each set at least once |
| cg_stretch | Clock stretch at SCL-low-to-high transition; stretch immediately after START |
| cg_fsm | All FSM states visited; all transitions exercised |
| cg_fifo | TX and RX FIFO: empty, half-full, and full conditions |
| cg_int | Each interrupt source independently masked and unmasked while active |

9.6  Formal Verification

Prove FSM cannot reach undefined state encoding from any reachable state.

Prove sda_oen and scl_oen are never simultaneously asserted to drive logic-1 (structural impossibility by design).

Prove that after presetn assertion for ≥ 2 cycles, all outputs reach reset values within 1 cycle.

Prove no state transition occurs when pclk is toggling but presetn is asserted.

10  Compliance Reference

10.1  Normative References

| Reference | Title | Version | Source |
| --- | --- | --- | --- |
| NXP UM10204 | I²C-bus specification and user manual | Rev. 7, Oct 2021 | www.nxp.com |
| AMBA APB | AMBA 3 APB Protocol Specification | ARM IHI0024C | developer.arm.com |
| IEEE 1800 | SystemVerilog Language Reference Manual | IEEE 1800-2017 | standards.ieee.org |
| UVM 1.2 | Universal Verification Methodology | IEEE 1800.2-2020 | accellera.org |

10.2  Compliance Checklist

The following I²C specification requirements have been addressed in this design:

| Req. ID | UM10204 Section | Requirement Summary | Status |
| --- | --- | --- | --- |
| REQ-01 | 3.1.1 | SDA changes only when SCL is LOW | Implemented |
| REQ-02 | 3.1.1 | START: SDA HIGH-to-LOW while SCL HIGH | Implemented |
| REQ-03 | 3.1.1 | STOP: SDA LOW-to-HIGH while SCL HIGH | Implemented |
| REQ-04 | 3.1.3 | ACK: 9th clock pulse; receiver pulls SDA low | Implemented |
| REQ-05 | 3.1.7 | Clock stretching detection and support | Implemented |
| REQ-06 | 3.1.8 | Arbitration: bitwise SDA comparison | Implemented |
| REQ-07 | 3.1.9 | Clock synchronization for multi-master | Implemented |
| REQ-08 | 3.1.11 | 10-bit addressing: two-byte sequence | Implemented |
| REQ-09 | Table 10 | Sm timing parameters (t_HD, t_LOW, t_HIGH ...) | Implemented |
| REQ-10 | Table 10 | Fm timing parameters | Implemented |
| REQ-11 | Table 10 | Fm+ timing parameters | Implemented (optional) |
| REQ-12 | 7.1 | Open-drain output topology | Implemented (I/O cell contract) |
| REQ-13 | 3.1.6 | Glitch filtering on SDA/SCL inputs | Implemented (3-cycle filter) |
| REQ-14 | 3.1.1 | Bus-free time (t_BUF) enforced before new START | Implemented |
| REQ-15 | 3.1.12 | General Call address (0x00) detection | Implemented (pass-through) |

10.3  Known Limitations and Deviations

High-speed mode (3.4 Mbit/s) and Ultra-fast mode (5 Mbit/s) are not implemented. Future revisions may add Hs-mode with current-source pull-up emulation.

Slave mode is not implemented in Revision A. A separate slave controller IP block is planned.

SMBus (System Management Bus) extensions (alert response, host notify, SMBSUS) are out of scope.

PMBus protocol extensions are not implemented.

The controller does not implement hardware I²C address decoding for slave-mode operation.

10.4  Document Control

This document is controlled under the project configuration management system. All changes require a design review by at least two hardware architects and sign-off from the lead verification engineer before a new revision is released. The authoritative copy is maintained in the project document repository.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| RTL Design Lead | TBD | _______________ | March 2026 |
| Verification Lead | TBD | _______________ | March 2026 |
| Architecture Review | TBD | _______________ | March 2026 |

HDS-I2C-001 Rev A.1  |  Confidential  |  © 2026 RTL Design Team